SAMSUNG

LCV-30A/B

VERTICAL MACHINING CENTER

MACHINE CONSTRUCTION / FEATURES

MAIN FRAME

- Lowering center of gravity design and secured wide bed guide way

- Hardened and ground box type slideway

- High rigid triangular rib body structure minimize vibration and deformation under heavy machining

SPINDLE  HEAD

| Spindle Taper | Unit | BT 30 |  |
| --- | --- | --- | --- |
| Spindle Drive Motor |  | αiI 3/10000 | αiI 2/20000 |
| Spindle speed | rpm | 10,000 | 15,000 |
| Spindle Drive Method | type | Direct Drive | Directly connected |
| Motor power | kW | 5.5[30min]/3.7 | 3.7[30min]/2.2 |
| Max. Torque (S3 25%/Short/ Cont) |  | 35.3/23.5 | 11.8/6.9 |
| Spindle Bearings | Front | 7010CD/P4AQBCA | 7020CD/P4AQBCA |
| Spindle BRG lubrication |  | Grease | Grease |
| Spindle cooling (Oil Cooling) | - |  |  |
| Tool clamping Method | - | Disk Spring(MUBEA) | Disk Spring(MUBEA) |
| Tool clamping/unclamping force | N | 1,486.8 / 1,987.9 | 1,486.8 / 1,987.9 |
| Tool unclamping method | kgf/㎠ | Pneumatic 5 | Pneumatic 5 |

SPINDLE POWER-TORQUE DIAGRAM

BT30 10000 RPM

Power (kW) Torque (N.m) RPM

BT30 15000 RPM

RPM Torque (N.m) Power (kW)

POWERFUL Fanuc α Series

SERVO DRIVE SYSTEM

Both side fixed type supporting system (2x4rows thrust angular ball bearings)

Pre-tensioned Double anchor method ball screw supporting system

| Items |  | X-axis | Y-axis | Z-axis |
| --- | --- | --- | --- | --- |
| Travel Rapid Traverse |  | 500mm 48m/min | 320mm 48m/min | 330mm 48m/min |
| Ball screw | Diameter x Lead | Ø32mm x 16mm | Ø32mm x 16mm | Ø32mm x 16mm |
|  | Spec.(HIWIN) | R32-16K5-FSC-692-838-0.008 | R32-16K5-FSC-520-693-0.008 | R32-16K5-FSC-523-705-0.008 |
| Drive motor (FANUC) | Servo Motor Type Rated power | α 8/3000i 1.6kW | α 8/3000i 1.6kW | α 8B/3000i 1.6kW |
| Feedback system | Absolute |  |  |  |
| Guide way | Slide Ways Slide Way Span Slide way Width Safety Device | L.M guide 450mm 34mm Soft OT, Mech. Stopper | L.M guide 300mm 34mm Soft OT, Mech. Stopper | L.M guide 350mm 34mm Soft OT, Mech. Stopper |
|  | Spec.(HIWIN) | HGW35HAE1R1010ZBP II | HGW35HAE2R800ZBP II | HGH30HAE2R790ZBP II |
| Accuracy | Positioning Repeatability | ±0.003 ±0.002 |  |  |

AUTOMATIC TOOL CHANGER

/ MAGAZINE

- Tool Shank < BT 30 >

- Pull stud type	< MAS P30T-1 > - 45° type

- Maximum tool Diameter		75 mm

- Maximum tool Diameter     		80 mm

[Adjacent empty]

- Maximum tool length     		200 mm

- Maximum tool weight	     		3 kgf

- Tool changing method	     		Double arm swing

- Tool selection method	     		Memory random

- Tool changing time(T-T)		0.7 sec

- Magazine capacity            	16 ea

TABLE

- Table size		            	500mmx1,000mm(*660mmx400mm)

- Loading capacity			150x150kgf(*Fixed table)

- Table surface Std.-Tap type

40-M16x100x100

- Table surface Opt.-T-slot

Width		18 mm

No. of slot 	8ea(*3ea)

Distance between T-slot	100(*110) mm

- Distance from Floor to Table surface	1,040(*1,019) mm

< LCV-30A >

<LCV-30B>

LOADING CAPACITY DIMENSION

LCV-30A

LCV-30B

HYDRAULIC POWER UNIT(LCV-30A Only)

- Motor power 						2.2 kW

- Discharge setting pressure				3.43 Mpa

- Max. discharge flow rate					28 L/min (60Hz)

- Tank capacity						24 L

LUBRICATION

- Pump unit						AMZ-III-100S-30LP (LUBE)

- Motor						18/19W  AC100V x 50 / 60 Hz

- Discharge flow rate					0.09 / 0.11 L/min

- Discharge pressure					1.47 Mpa

- Tank capacity						3 Liters

PNEUMATIC

- F.R.L unit 						AWL4000-G04BG-X (SMC)

- operating Pressure 					0.5 Mpa

- Air Lubricator oil						ISO VG-32

COOLANT  AND  CHIP PAN

- Type							Removable / Independent

- Coolant capacity						190 Liter

- Pump motor						1500 W

- Discharge & Pressure					400 L/min

MACHINE LAY-OUT ( LCV-30A )

MACHINE LAY-OUT (LCV-30B)

MACHINE SPECIFICATIONS

TRAVEL

X axis travel						500 mm

Y axis travel				320 mm

Z axis travel				330 mm

Distance from table top to spindle nose	150-480(*220-550) mm

Distance from column to spindle center		397mm

TABLE

Table size						500x1,000(*600x400) mm

Loading capacity			150 x 150kgf

Table surface configuration		40-M16x100x100(*32-M16x80x80)

SPINDLE

Spindle speed						10,000 rpm

Bearing inner Diameter			Ô50 mm

Max. Torque (30min/cont.)		35.3/23.5

AUTO TOOL CHANGER

Tool Shank					BT30

Pull stud type			45° type

Maximum tool Diameter	[Adjacent empty]    		75 mm  [80 mm ]

Maximum tool length  			    		200 mm

Maximum tool weight	     				3 kgf

Tool changing method	   		   		Double arm swing

Tool selection method	  		   		Memory random

Tool changing time(T-T)					0.7 sec

Magazine capacity 			           	16 ea

FEED RATE

Rapid traverse(X/Y/Z)					48/48/48 m/min

Cutting feed rate			                  1~10,000mm

ACCURACY

Positioning				±0.003

Repeatability				±0.002

MOTORS

Spindle drive motor					5.5[30min]/3.7

Feed motor (X/Y/Z)			1.6kw/1.6kw/1.6kw

Coolant pump motor			1500 W

Lubricant pump motor 			18 W

MACHINE SIZE

Floor space (LxW)					3,578x1,470(3,178x1,465) mm

Machine height				2,145(2,095) mm

Mass of machine                                   3,200(3,100) kgf

Total power capacity					20kVA

CNC System							Fanuc 0i-MD,

STANDARD EQUIPMENT

1. Full Splash guard

2. Coolant tank

3. Machine installation parts

4. Standard tool and tool box

5. Friction oil self-oiling system

6. Work lamp

7. Call light (3 color)

8. Rigid tapping

9. Spindle override

10. Bed flushing

CONTROL SPECIFICATIONS

|  | Item |  |
| --- | --- | --- |
| Controlled axis | Controlled axes | 3(X,Y,Z) |
|  | Max. simultaneously controlled axis | 4 |
|  | Least command increment | 0.001mm / 0.0001" |
|  | Stored stroke check 1 | O |
|  | Stored stroke check 2,3 | O |
| Interpolation/Feed functions | Linear interpolation | G01 |
|  | Circular interpolation | G02, G03 |
|  | Dwell | G04 |
|  | Cylindrical interpolation | G07.1 |
|  | Nano smoothing | OPT |
|  | Reference position return | G28 |
|  | Reference position return check | G27 |
|  | 2nd/3rd/4th reference position return | G30 |
|  | Rapid traverse rate override | F0, 25%, 50%, 100% |
|  | Feedrate override | 0~200% |
|  | Jog Override | O |
|  | AI contour control I | - |
|  | AI contour control II | O |
|  | Pulse handle feed | X1, X10, X100 |
|  | Feedrate per minute | G94 |
|  | Feedrate per revolution | G95 |
| Spindle function | Spindle orientation | O |
|  | Rigid tapping | O |
|  | Spindle override | 50 ~ 150% |
| Tool functions | Tool number command | T2-Digt |
|  | Tool nose radius compensation | G40 ~ G42 |
|  | Tool offset pairs | 400 |
|  | Tool geometry/wear offset | O |
|  | Tool life management | O |
|  | Tool path graphic display | O |
|  | Automatic tool length measurement | O |
| Program input, Editing operation | Absolute/incremental programming | O |
|  | Multiple repetitive cycle | G70 ~ G76 |
|  | Canned cycles | G90, G92, G94 |
|  | Decimal point programming | O |
|  | Inch/metric conversion | G20 / G21 |
|  | Program restart | O |
|  | Sub program call | O |
|  | Retraction for rigid tapping | O |
|  | Max. programmable dimension | ±99999.999mm/±9999.9999" |
|  | M function | M3 digit |
|  | Custom macro | O |
|  | Canned cycle for drilling | O |
|  | Direct drawing dimension programming | O |
|  | Programmable data input | G10 |
|  | Tape code | ISO / EIA |
|  | Single Block | O |
|  | Dry Run | O |
|  | Optional block skip | O |
|  | Workpiece coordinate system | G52 ~ G59 |
|  | Part program storage size | 512Kbyte(1280m) |
|  | Number of registerable programs | 400EA |
| Interface function, Data input/output | Embedded ethemet | O |
|  | I/O Interface | O |
|  | Fast data server | O |
|  | Memory card input/output | O |
|  | USB memory input/output | O |
| Setting and display | Alarm & Operator histor display | O |
|  | Run hour and parts count display | O |
|  | Display spindle & servo overload | O |
|  | Self-diagnosis function | O |
|  | Extended part program editing | O |
|  | Display screen | 8.4"/10.4" color |
|  | External message | O |
|  | Multi-language display | O |

INVESTMENT

BASE PRICE:						$__________ FOB        PORT

SAMSUNG LCV30A/B with Fanuc 0i-MD control

TERMS OF PAY	MENT:

OPTIONAL EQUIPMENT

1. Chip conveyor (Side, Rear : Lift up type, Scraper type, Mesh drum type)

2. Air blow

3. Gun coolant

4. Auto tool length measuring device

7. Addition 1-axis, 2-axis device

8. 3MPG

9. Work Counter

10. Integrating meter

11. Data server

12. AICC

13. Oil skimmer

14. Auto power cut off

STANDARD TERMS AND CONDITIONS

Warranty

The machine is warranted against defects in parts, material and workmanship for a period of twelve months after the date of installation.  The control, servomotors and main spindle motor are warranted against defects in parts, material and workmanship for a period of twenty-four months after the date of installation.

Installation

Installation of the machine/control system is supervised by SAMSUNG factory trained servicemen without charge to customer.  Initial training on machine/control operation and programming will also be done at the time of the machine installation.  Additional training in N/C programming will be available on a scheduled basis.

Delivery

Free on Board, Port of Entry, Duty paid, approximately two to four months upon receipt of firm purchase order with down payment.

Payment Terms

(                       )

Thank you for the opportunity to quote your machining needs with the SAMSUNG LCV-650 Machining Center.

After an examination of this quotation, should you have any questions, or desire additional information, please do not hesitate to contact us.

Sincerely,