SAMSUNG

LCV 380D / 380S

VERTICAL MACHINING CENTER

MACHINE CONSTRUCTION / FEATURES            (*  ) : Spec. of LCV 380S

MOVABLE COLUMN TYPE VERTICAL

Tap Center “LCV 380 Series”

- Direct spindle 12,000rpm for high accuracy tapping.

- Applying high precision roller guide on all axis(X,Y,Z)

- Rapid travers 50m/min, reducing machine idle time with 3kw motor

- Adopting synchronized high precision roller gear cam tool change system, LCV 380 has the fastest tool change time in its class. (T-T0 .85 Sec)

SPINDLE  HEAD

P4 class high precision angular contact ball bearings are used for high speed and ultra precision machining.

Direct drive main spindle for high speed-high accuracy synchrorized tapping.

| Spindle Taper | Type | BT30 |  |
| --- | --- | --- | --- |
| Spindle Drive Method | Type | Direct drive | Direct drive |
| Spindle speed | RPM | 12000 | 15000 |
| Spindle Drive Motor |  | αiI 3/12000 | αiI 2/20000 |
| Motor power(30min/Cont.) | kW | 5.5 / 3.7 | 3.7 / 2.2 |
| Max. Torque (S3 25%/Short/ Cont) | N.m | 35.3 / 23.5 | 11.8 / 6.9 |
| Spindle Bearings | Front | 50BNR10STYNDBBELP4Y | 50BNR10HTYNDBBELP4Y |
| Spindle coupling | - | ROTEX GS-P 38(D44G6/D28H6) | ROTEX GS-P 38(D44H6/D22G6) |
| Spindle BRG lubrication | Type | Grease | Grease |
| Spindle cooling (Oil Cooling) | - | Opt. | Opt. |
| Tool clamping Method | - | Disk spring(MUBEA) |  |
| Tool clamping/unclamping force | N | 3186 / 4569 | 2540 / 4421 |
| Tool unclamping method | kgf/㎠ | Pneumatic 5 | Pneumatic 5 |

SPINDLE POWER-TORQUE DIAGRAM

Power (kW)

Power (kW)

Torque (N.m)

RPM

RPM

Torque (N.m)

SERVO DRIVE SYSTEM

Guide plane of roller guide

High precision machining is achieved with the widest guide span in its class, low-centered box structure design, high-load roller guide.

Ball screws (X,Y,Z)

“Fixed and Supported” structure is applied on 32mm ball screws to minimize the change of ball screws in high speed machining and it provides extended life of ball screws also long lasting high precision.

3kw large capacity motor (X,Y,Z)

3kw motor is applied on all axis to achieve quiet and smooth machining and highly accurate cornering.

| Items |  | X-axis | Y-axis | Z-axis |
| --- | --- | --- | --- | --- |
| Travel Rapid Traverse |  | 520mm 50m/min | 380mm 50m/min | 350mm 50m/min |
| Ball screw | Diameter x Lead | Ø32mm x 20mm | Ø32mm x 20mm | Ø32mm x 20mm |
|  | Spec.(HIWIN) | R32-20K5-FSC-714-880-0.018 | R32-20K5-FSC-557-720-0.018 | R32-20K5-FSC-525-713-0.018 |
|  | Opt.(NSK) | W3207P-61PSS-C5Z20 | W3205P-84PSS-C5Z20 | W3205P-85PSS-C5Z20 |
| Drive motor (FANUC) | Servo Motor Type Rated power | ß iS22/3000 3kW | ß iS22/3000 3kW | ß iS22B/3000 3kW |
| Feedback system | Absolute |  |  |  |
| Guide way | Slide Ways Slide Way Span Slide way Width Safety Device | LM Guide 650mm 34mm Soft OT, Mech. Stopper | LM Guide 370mm 34mm Soft OT, Mech. Stopper | LM Guide 310mm 28mm Soft OT, Mech. Stopper |
|  | Spec.(HIWIN) | RGW35HC2R1078ZBP II | RGW35CC2R958ZBP II | RGH30CA2R8082BP II |
| Accuracy | Positionig Repeatability | ± 0.005 ± 0.002 |  |  |

AUTOMATIC TOOL CHANGER

/ MAGAZINE

- Tool Shank				BT30

- Pull stud type				MAS P30T-1(45° type)

- Maximum tool Diameter		60 mm

- Maximum tool Diameter     		120 mm

[Adjacent empty]

- Maximum tool length     		200 mm

- Maximum tool weight	     		3 kgf

- Tool changing method	     		Double arm swing

- Tool selection method	     		Memory random

- Tool changing time(T-T)		0.85 sec

- Magazine capacity            	20 ea

TABLE

DUAL TABLE - LCV 380D / TABLE - LCV 380S

- Table size		            	2-600x520 (*600x420) mm

- Loading capacity			2x200 (*Fixed table) kgf

- Table surface Std.-Tap type

- LCV 380D		2x33-M16x90x90

- LCV 380S		35-M16x80x80

- Table surface - Opt. T-slot

Width					14

No. of slot 				6(*3) ea

Distance between T-slot		110 mm

Distance from Floor to Table surface	950(*930) mm

LOADING CAPACITY DIMENSION

LCV 380D

LCV 380S

HYDRAULIC POWER UNIT(LCV 380D Only)

- Motor power 						1.5 kW

- Discharge setting pressure				60 kgf/㎠

- Max. discharge flow rate					14 L/min (60Hz)

- Tank capacity						11 L

- Main power						1.5kWx200V/380x4Px50/60Hz

LUBRICATION

- Pump unit						AMZ-III-100S-30LP (LUBE)

- Motor						18W  AC100V x 60Hz

- Discharge flow rate					0.09 / 0.11 L/min

- Discharge pressure					15 kgf/㎠

- Tank capacity						3 Liters

PNEUMATIC

- F.R.L unit 						AW40-04DG (SMC)

- operating Pressure 					0.5~0.6 Mpa

- Air Lubricator oil						ISO VG-32

COOLANT  AND  CHIP PAN

- Type							Removable / Independent

- Coolant capacity						327 Liter

- Pump motor						400 W

- Discharge & Pressure					200 L/min

0.5 kg/cm2  60Hz

MACHINE LAY-OUT(LCV 380D)

MACHINE LAY-OUT(LCV 380S)

MACHINE SPECIFICATIONS

TRAVEL

X axis travel						520 mm

Y axis travel				380 mm

Z axis travel				350 mm

Distance from table top to spindle nose	200~550(*570) mm

TABLE

Table size						2-600x520(*600x420) mm

Loading capacity			2x200(*Fixed table) kgf

Table surface configuration		2x33-M16x90x90(*35-M16x80x80)

SPINDLE

Spindle speed						12,000 rpm

Transmission				Direct drive

Bearing inner Diameter			ø50 mm

Max. Torque (30min/cont.)		3.6/2.4 kgf.m [35.3/23.5 N.m]

AUTO TOOL CHANGER

Tool Shank					BT30

Pull stud type			MAS P30T-1 (45° type)

Maximum tool Diameter	[Adjacent empty]    		60 mm [120 mm]

Maximum tool length  			    		200 mm

Maximum tool weight	     				3 kgf

Tool changing method	   		   		Double arm swing

Tool selection method	  		   		Memory random

Tool changing time(T-T)					0.85 sec

Magazine capacity 			           	20 ea

FEED RATE

Rapid traverse(X/Y/Z)					20/20/16 m/min

Cutting feed rate			1~10,000mm

MOTORS

Spindle drive motor(30min/Cont.)			5.5/3.7kw

Feed motor (X/Y/Z)			3kw/3kw/3kw

Coolant pump motor			400 W

Lubricant pump motor 			18W

MACHINE SIZE

Floor space (LxW)				3,476x1,898(*3,190x1,870) mm

Machine height			2,443 mm

Mass of machine                                  	4,500(*4,000) kgf

Total power capacity				30 kVA

CNC System							Fanuc 0i-MC

STANDARD EQUIPMENT

- 8.4” TFT LCD Color Monitor

- Machine Work Light : halogen Lamp

- Coolant System: Tank capacity: 327 Liters, 400 W pump

- Full Splash Guard

- Hydraulic Unit(LCV 380D Only)

- Lubrication System

- Machine installation part

- Manuals: (Instruction, Operating, Part List, Electric circuit diagrams)

CONTROL SPECIFICATIONS

- TYPE OF CONTROL

- 32-bit multiprocessor continuous-path control

- HARDWARE COMPONENTS

- 8.4” Color  TFT LCD screen

- Flash memory card interface

- RS 232C interface

- SCREEN DISPLAY

- Window oriented operator interface

- Display of current block during program execution

- Screen texts: English (other languages: Option)

- Actual cutting / spindle speed display

- Alarm display and Alarm history display

- Clock (function) display

- OPERATION

- Program protection on machine control panel

- 640M part programming storage

- Number of Registered Programs: 400

- Built-in Run Hour / Parts Counter Display

- Tool life management

- Automatic tool measurement and compensation (with tool eye)

- Background editing

- Linear and circular interpolation

- Help function

- MODES

- AUTOMATIC

- Control of AUTOMATIC mode by:

- Feed hold and spindle stop

- Skip block

- Single block

- Dry run feedrate

- JOG (setup)

- MDI (manual data input)

- MACHINE CONFIGURATION FOR AXES

- Inch or metric programming

- Switchover between metric and inch for input and offsets, display, programmed traverse path

- Feed-rate and rapid traverse: minimum input feed-rate in mm/min=0.001(inches/min = 0.0001) .

- Revolution feed-rate: minimum input federate in mm/rev=0.001( inches/rev = 0.0001)

- MACHINE SPINDLE CONFIGURATIONS

- Constant cutting speed, cutting feed-rate clamp

- 1 position Spindle Orientation

- OVERRIDES , OFFSETS AND COMPENSATIONS

- Feed-rate override 0% to 200%

- Rapid traverse override 0%, 25%, 50%, 100%

- Spindle speed override 50% to 120%

- Tool Offsets(400pairs)

- Backlash compensation

- Cutter compensation C

- CNC PROGRAMMING

- Mirror image by each axis

- Stored stroke check 1, 2nd reference position return

- Work coordinate system (G52 ~ G59)

- Skip function, optional block skip / 1 ea

- Sub-program call (4 level)

- Searching function: Program no., Sequence no., and external work no.

- Absolute and incremental programming

- Dwell time can be programmed in seconds or revolutions

- Single block by each axis, dry run

- Program restart

- Rigid tapping

- Number of registered program(400)

- Parts program storage length(640M)

- Custom macro B

- Optional block skip 9

- SAFETY AND DIAGNOSTIC FUNCTIONS

- Safety routines permanently active for measuring circuits, over temperature, battery, voltage, memory, limit switches, fan monitoring

- Self diagnostics

- Contour monitoring

- Spindle monitoring

INVESTMENT

BASE PRICE:						$__________ FOB        PORT

SAMSUNG LCV 80 / 66 with Fanuc 0i-MD control

TERMS OF PAYMENT:

OPTIONAL EQUIPMENT

- Lift up chip conveyor & Bucket

- Gun coolant

- Air blow

- Through spindle coolant

- Oil skimmer

- Auto tool length measurement unit

- Additional 1 axis,2Axis system

- 3-MPG

- Data server

- AI contour control

- Air gun

- Side oil hole

- Flood coolant

STANDARD TERMS AND CONDITIONS

Warranty

The machine is warranted against defects in parts, material and workmanship for a period of twelve months after the date of installation.  The control, servomotors and main spindle motor are warranted against defects in parts, material and workmanship for a period of twenty-four months after the date of installation.

Installation

Installation of the machine/control system is supervised by SAMSUNG factory trained servicemen without charge to customer.  Initial training on machine/control operation and programming will also be done at the time of the machine installation.  Additional training in N/C programming will be available on a scheduled basis.

Delivery

Free on Board, Port of Entry, Duty paid, approximately two to four months upon receipt of firm purchase order with down payment.

Payment Terms

(                       )

Thank you for the opportunity to quote your machining needs with the SAMSUNG LCV-650 Machining Center.

After an examination of this quotation, should you have any questions, or desire additional information, please do not hesitate to contact us.

Sincerely,